TRACER-MIP WRF-Chem output data description

This dataset contains WRF-Chem model output processed for the TRACER-MIP intercomparison. The variables were extracted from the original WRF-Chem wrfout files following the TRACER-MIP requested variable list as closely as possible. This dataset contains WRF-Chem model output processed for the TRACER-MIP intercomparison. The variables were extracted from the original WRF-Chem wrfout files following the TRACER-MIP requested variable list as closely as possible. The current export includes nearly all requested TRACER-MIP variables that are available from the WRF-Chem output, including atmospheric state variables, water and hydrometeor mixing ratios and number concentrations, precipitation, surface fields, radiation fluxes, aerosol mass/number-related fields, and most microphysical process-rate diagnostics. Two categories are incomplete or not directly included in the current export: explicit latent heating/cooling tendency variables, and aerosol effective or median radius variables. Some latent-heating-related information can still be inferred from the exported microphysical process-rate variables, but the direct heating/cooling diagnostic itself is not currently included.

The model simulations were conducted at three horizontal grid spacings: 3 km, 1 km, and 500 m (Figure1). The 1 km simulations are the primary dataset, as listed in Table 1, include ndust-nPBAP, dust-nPBAP, ndust-PBAP, ndust-PBAP-rup, dust-PBAP, dust-PBAP-rup. Each experiment was performed with three ensemble members generated by applying small, horizontally smoothed random perturbations to the initial boundary-layer potential temperature field below the simulated PBLH. The perturbations had a target variance of approximately 0.05 K$^2$, corresponding to a standard deviation of about 0.22 K.

Figure.1 (a) Spatial distribution of leaf area index (LAI) from the outer domain, with the red box denoting the 1 km inner nested domain. (b) Mean fungal spore emission flux over the 1 km domain on 17 June 2022, overlaid with the monitoring sites used in this study. Yellow stars denote the ARM S3 and M1 sites, red circles mark TCEQ monitoring stations, and green triangles indicate EPA sites. (c) Comparison of simulated PBAP number concentrations with observations from the Houston Health Department. Shaded distributions represent multi-year mean observation for 2018–2022, with blue indicating the full June multi-year mean and green indicating the June 16–20 multi-year mean. Boxplots show the observed median (solid line) and mean (dashed line). Blue markers denote observed daily means reported for the City of Houston on 16 June 2022. Purple markers denote the simulated mean PBAP concentration averaged over the case period (12:00 June 17 to 06:00 June 18, 2022, UTC) sampled from the grid cell corresponding to the City of Houston, and orange markers represent the 1 km domain-mean simulated PBAP concentrations averaged over the same period.

The 3 km and 500 m outputs are provided only for the ndust-nPBAP baseline case. The model output frequency is hourly. The nested simulations were performed using one-way nesting and/or ndown. Chemical initial and boundary conditions for the 3 km domain were derived from CAM-Chem. Meteorological initial and boundary conditions were driven by NAM analysis data.

Anthropogenic emissions were based on the NEI2017 emission inventory, and biogenic emissions were calculated using MEGAN. In this study, simulated PBAPs refer to fungal-spore-related biological particles and their ruptured fragments represented in the model. The fungal spore emission flux was calculated offline following Janssen et al. (2021), using meteorological inputs from the FDDA-constrained WRF simulation, including 2~m specific humidity and friction velocity, and LAI from the WRF land-surface fields (Figure 1b).

## For 1hr output data, we have:

## Core state and precipitation variables

| Variable | Meaning | Typical dimensions | Units |
| --- | --- | --- | --- |
| pressure | Total air pressure from P + PB | Time, bottom_top, south_north, west_east | hPa |
| height | Geopotential height on mass levels | Time, bottom_top, south_north, west_east | m |
| air_density | Air density computed from pressure and virtual temperature | Time, bottom_top, south_north, west_east | kg m-3 |
| U | Zonal wind, destaggered to mass grid | Time, bottom_top, south_north, west_east | m s-1 |
| V | Meridional wind, destaggered to mass grid | Time, bottom_top, south_north, west_east | m s-1 |
| W | Vertical wind, destaggered to mass grid | Time, bottom_top, south_north, west_east | m s-1 |
| precip_accum | Accumulated precipitation from RAINC + RAINNC | Time, south_north, west_east | mm |
| precip_rate | Precipitation rate from change in accumulated precipitation | Time, south_north, west_east | mm s-1 |

## Static map fields

| Variable | Meaning |
| --- | --- |
| HGT | Terrain height |
| XLAT | Latitude on unstaggered or native grid |
| XLONG | Longitude on unstaggered or native grid |
| XLAT_M | Latitude on mass grid |
| XLONG_M | Longitude on mass grid |

## Radiation fluxes

| Variable | Meaning |
| --- | --- |
| SWDNB | Downwelling shortwave flux at surface |
| SWUPB | Upwelling shortwave flux at surface |
| LWDNB | Downwelling longwave flux at surface |
| LWUPB | Upwelling longwave flux at surface |
| SWDNT | Downwelling shortwave flux at top of atmosphere |
| SWUPT | Upwelling shortwave flux at top of atmosphere |
| LWDNT | Downwelling longwave flux at top of atmosphere |
| LWUPT | Upwelling longwave flux at top of atmosphere |

## Surface time-varying fields

| Variable | Meaning |
| --- | --- |
| HFX | Sensible heat flux |
| LH | Latent heat flux |
| ALBEDO | Surface albedo |
| SLP | Sea level pressure |

## Water and hydrometeor mixing ratios

| Variable | Meaning |
| --- | --- |
| QVAPOR | Water vapor mixing ratio |
| QCLOUD | Cloud water mixing ratio |
| QRAIN | Rain water mixing ratio |
| QICE | Cloud ice mixing ratio |
| QSNOW | Snow mixing ratio |
| QGRAUP | Graupel mixing ratio |

## Hydrometeor number concentrations

| Variable | Meaning |
| --- | --- |
| QNCLOUD | Cloud droplet number concentration |
| QNRAIN | Rain number concentration |
| QNICE | Ice number concentration |
| QNSNOW | Snow number concentration |
| QNGRAUPEL | Graupel number concentration |

## Aerosol mode and bulk fields

| Variable | Meaning |
| --- | --- |
| nu0 | Aerosol mode number field |
| ac0 | Aerosol mode number field |
| corn | Aerosol mode number field |
| antha | Aerosol bulk mass field as named in the current model output |
| seas | Sea salt aerosol bulk mass field |
| soila | Soil aerosol bulk mass field |
| polp | Primary pollen aerosol bulk mass field |
| pols | Secondary pollen aerosol bulk mass field |

## Aerosol species family fields

| Prefix or suffix | Meaning |
| --- | --- |
| so4 | Species prefix searched by exporter |
| nh4 | Species prefix searched by exporter |
| no3 | Species prefix searched by exporter |
| na | Species prefix searched by exporter |
| cl | Species prefix searched by exporter |
| org | Species prefix searched by exporter |
| ec | Species prefix searched by exporter |
| p25 | Species prefix searched by exporter |
| i | Suffix included when the corresponding raw variable exists |
| j | Suffix included when the corresponding raw variable exists |
| cwi | Suffix included when the corresponding raw variable exists |
| cwj | Suffix included when the corresponding raw variable exists |

## PM fields

| Variable | Meaning |
| --- | --- |
| PM2_5_DRY | Dry PM2.5 |
| PM10 | PM10 |

## Additional aerosol number fields

| Variable | Meaning |
| --- | --- |
| NPOLICE | Aerosol number-related field from raw WRF-Chem output |
| NDUSTICE | Aerosol number-related field from raw WRF-Chem output |
| NECICEI | Aerosol number-related field from raw WRF-Chem output |
| NECICEJ | Aerosol number-related field from raw WRF-Chem output |
| NIAER | Aerosol number-related field from raw WRF-Chem output |

## Exported microphysics process variables

| Variable | Short description from the export metadata |
| --- | --- |
| PRC | Cloud water autoconversion |
| PRA | Rain-cloud water collection |
| NPRC | Cloud water autoconversion, number |
| NPRA | Rain-cloud water collection, number |
| NRAGG | Rain self-collection or breakup |
| MNUCCC | Ice from contact and immersion freezing, mass |
| MNUCCD | Ice from deposition freezing, mass |
| ICE_NUCH | Homogeneous ice nucleation from aqueous aerosol |
| NNUCCC | Ice from contact and immersion freezing, number |
| PSACWI | Ice-cloud water collection, mass |
| PRACI | Ice-rain collection, mass |
| NPSACWI | Ice-cloud water collection, number |
| NIACRS | Ice-rain collection, number |
| PSACWS | Snow-cloud water collection, mass |
| PRAI | Snow-cloud ice collection, mass |
| PRACS | Snow-rain collection, mass |
| PRACIS | Ice-rain collection to snow, mass |
| NPSACWS | Snow-cloud water collection, number |
| NPRAI | Snow-cloud ice collection, number |
| NPRACS | Snow-rain collection, number |
| NSAGG | Snow self-collection, number |
| PSACWG | Graupel-cloud water collection, mass |
| PRACG | Graupel-rain collection, mass |
| PGSACW | Snow-cloud water to graupel, mass |
| PGRACS | Rain to graupel, riming mass |
| NPSACWG | Graupel-cloud water collection, number |
| NPRACG | Graupel-rain collection, number |
| NSCNG | Snow-cloud water to graupel, number |
| NGRACS | Rain to graupel, number |
| QMULTS | Snow riming cloud water to new ice, mass |
| QMULTR | Snow riming rain to new ice, mass |
| QMULTG | Graupel riming cloud water to new ice, mass |
| QMULTRG | Graupel riming rain to new ice, mass |
| NMULTS | Snow riming cloud water to new ice, number |
| NMULTR | Snow riming rain to new ice, number |
| NMULTG | Graupel riming cloud water to new ice, number |
| NMULTRG | Graupel riming rain to new ice, number |
| PCC | Liquid condensation or evaporation tendency |
| PCCN | Droplet activation tendency |
| PRE | Rain evaporation tendency |
| PRD | Deposition growth of cloud ice |
| PRDS | Deposition growth of snow |
| PSMLT | Melting snow to rain |
| PGMLT | Graupel melting or sublimation term as stored in model output |

## For 5min output data (only 1km and 500m simulations), we have:

tobac_ref3d-2d_20220617.nc

| Variable | Dimensions | Units | Description |
| --- | --- | --- | --- |
| time | time | seconds since 1970-01-01 00:00:00 | Time coordinate |
| z | z | m | Layer height; domain-mean, MSL |
| lat | y, x | degrees_north | Latitude |
| lon | y, x | degrees_east | Longitude |
| reflectivity | time, z, y, x | dBZ | Radar reflectivity at 10 cm wavelength |
| w_colmax | time, y, x | m s-1 | Column maximum vertical velocity |
| rain_rate | time, y, x | mm h-1 | Rain rate from accumulated precipitation; step mean |
| rain_rate_1h | time, y, x | mm h-1 | One-hour rolling-mean rain rate from accumulated precipitation |
| rtop_height | time, y, x | m | Echo-top height, defined using reflectivity > 10 dBZ and rain_rate_1h > 0.01 |
| rtop_tempC | time, y, x | degC | Echo-top temperature at the echo-top level |
| rbase_height | time, y, x | m | Echo-base height, defined using reflectivity > 10 dBZ and rain_rate_1h > 0.01 |

tobac_w3d_20220617.nc

| Variable | Dimensions | Units | Description |
| --- | --- | --- | --- |
| time | time | seconds since 1970-01-01 00:00:00 | Time coordinate |
| z | z | m | Layer height |
| lat | y, x |  | Latitude |
| lon | y, x |  | Longitude |
| w | time, z, y, x | m s-1 | Vertical velocity on mass levels |